# Manual Classification

# Classification

The project seeks to classify abnormal patches within retinal images. Abnormal in this instance is defined by an area within the retinal image that is neither a blood vessel, nor the background. Specifically areas that appear darker than the surrounding background. Other abnormalities are not required for this project.

## Blood Vessels

Blood vessels, even if abnormally formed, are considered normal for this project.

All of the below are considered normal.

## Background

Retinal backgrounds, including those with small capillaries, are considered normal.

All of the below are considered normal.

## Abnormal

A background that appears darker than a normal background and is not obscured by blood vessels and does not contain a majority of normal background in the cell is considered abnormal.

All of the below are considered abnormal.

# Matlab

Scripts to generate manual classification have been written and are available on Github at https://github.com/MonkeyRum/CMP3060M_Item_1

To generate manual classification data:

- Download the repository [extract the zip if downloaded as an archive]

- Download the image data set from Dropbox at /CMP3060M/ImageDataSet/Original

- Add the repository scripts folder and the image data set to MatLab’s path

- Call the function ManuallyClassifyImageDataset with the following arguments

  - Full path to image directory with a trailing \

  - Full path to output directory with a trailing \

  - 256

  - 256

    - Eg  ManuallyClassifyImageDataset('F:\backup\Pictures\Imagedataset\', 'C:\manualClassification\', 256, 256)

  - Left click to toggle abnormal patch

  - Right click to finish marking

  - A CSV file will be output into the outDir

- Upload the manual classification to Dropbox at /CMP3060M/ImageDataSet/ManualClassification/<name>

https://www.dropbox.com/sh/n2qfoz6p833iz6f/AAB3x5VFUoBBszkLyH_DGMQRa?dl=0